Electrochemical determination of lead by laser -scripted reduced graphene oxide     electrode decorated with gold nanoparticles

Ana Ameda1,2

Entea Gjergji1,2, Andy-Bruno-Darder3, Ruslan Alvarez3, Majlinda Vasjari1,2, Arben Merkoci3

1Department of Chemistry, Faculty of Natural Science, University of Tirana, Bulevardi Zogu I, 1001 Tirane, Albania

2Nano-Alb, Academy of Sciences of Albania, Sheshi "Fan Noli", No 7, 1001 and Tirana, Albania

3ICN2, The Catalan Institute of Nanoscience and Nanotechnology, Autonomous University of Barcelona, Spain.

amedaana8@gmail.com

Over the past decade, surface water sources have been continuously polluted by harmful chemical substances, biological waste, and other contaminants, primarily due to rapid industrialization and human activities.[1] Among them, heavy-metal ions(HMIs) are a huge threat as they are very toxic, easy to accumulate, and non-degradable in the environment. The presence of lead ions (Pb2+) is associated with adverse effects on children's behavior, physical growth, cognitive skills, and educational achievement. [2] In this study, we present a graphene-based sensor for the detection of   Pb2+ in water. The sensor utilizes a conductive film composed of reduced graphene oxide (rGO) incorporated with gold nanoparticles (AuNPs), fabricated through a one-step, CO₂ laser-assisted co-reduction process. This method simultaneously reduces graphene oxide and gold cations to form the rGO@Au nanocomposite [3] Electrochemical characterization of –rGO@Au sensor was accomplished via Cyclic Voltammetry and Square wave anodic stripping voltammetry. [4] The sensor exhibited a sensitivity of 0.3367 µA/ppb, a 5-30 ppb linear range, and a correlation coefficient of 0.9941.

References

[1] N.Vasanthi.Sridharan.,et al., ACS Omega. (2022), 7, 45469-45480

[2] J. Hwang.,et al., Anal. Chem. ( 2019), 91, 11770-11777

[3] A. Scroccarello,et al., ACS Sens. (2023) , 8, 598−609

[4] Q. Yang,et al., ACS EST Water (2021), 1, 2470−2476