Reduced graphene oxide decorated with Fe2O3 nanoparticles composite carbon paste electrode (CPE)  for  voltammetric determination of  atenolol

Nevila Broli1,2

Ergi Hoxha2,3, Sadik Cenolli1,2, Majlinda Vasjari1,2

1Department of Chemistry, Faculty of Natural Science, University of Tirana, Bulevardi Zogu I, 1001 Tirana, Albania

2Nano-Alb, Academy of Sciences of Albania, Fan Noli 7, 1001 Tirana, Albania

3Western Balkans University, Faculty of Dental Medicine, Department of Basic Sciences,Tirana, Albania.

nevila.broli@fshn.edu.al

Abstract

An effective electrochemical sensing platform for the determination of atenolol (ATN)  based upon  magnetic nanoparticles Fe2O3/reduced graphene oxide (rGO) bulk modified carbon paste electrodes (F2O3-rGO/CPE) is reported. The electrochemical studies and measurements were carried out by using  voltammetry (CV) and square wave voltammetry (SWV) techniques. The nanocomposite-modified carbon paste electrode (Fe2O3-rGO/CPE) exhibited enhanced electrocatalytic performance for the determination of ATN. The proposed method using Fe2O3-rGO/CPE sensor is characterized by high sensitivity of measurements, with the linearity of ATN in the range from 40 to 3138 µmol L-1. The lowest detection limit achieved on the Fe2O3-rGO/CPE nanocomposite electrode for 10 s of preconcentration time was 58 µmol L-1 ATN in 0.1 mol L−1 H2SO4 of  pH 2.0 using square wave technique. The nanomodified Fe2O3-rGO/CPE  sensors showed a good reproducibility and repeatability (RSD ≤ 5%,) for ATN determination. The reduced graphene oxide decorated with magnetic  nanoparticles Fe2O3-rGO results with a good enhancement in the sensitivity of the sensor through a combination of increased surface area and improved electron transfer kinetics. Finally, the fabricated Fe2O3-rGO/CPE exhibits high sensitivity and good stability towards the sensing ANT and has the potential to be utilized as a clinical assay and quality assurance (QA) in pharmaceutical products.

Keywords: Atenolol, reduced graphene oxide, nanocomposite, modifed glassy carbon electrode, square wave  pulse voltammetry ·

References

- Joanna Smajdor1, Marcel Zambrzycki2, Mateusz Marzec3, Beata Paczosa-Bator1,Robert Piech1, Microchimica Acta (2023) Volume 190:449

- [1] Broli, N.,  Vasjari, M, Cenolli, Vallja, L.S. Duka, S, Shehu, A., (2021) , Opean Chemistry-19(1):875-0071

- Mona A Mohamed 1, Shimaa A Atty 1, Hanan A Merey 2, Taghreed A Fattah 1, Christopher W Foster 3, Craig E Banks 3, Analyst, Issue 19, (2017) page  3674-3679

- Eagambaram Murugan* and Kalpana Kumar.Analytical Chemistry. Vol 91/Issue 9. (2019)

Figures

Figure 1: Scematic determination of ATN with  Fe2O3-rGO/CPE sensor