Nanomachined tapered optical fibers for recording and controlling neural

Presenting Author: Massimo De Vittorio1,2

Co-Authors: Ferruccio Pisanello1

1Center for Biomolecular Nanotechnologies, Istituto Italiano di Tecnologia, Via Barsanti, Arnesano (LE), Italy

2Department of Health Technology, IDUN section, Technical University of Denmark, 2800 Kgs. Lyngby, Denmark

massimo.devittorio@iit.it

In this talk, new technological and methodological approaches for interfacing the brain by photonic technologies will be shown. Tapered optical fibers are nanomachined and processed to produce optical probes and optrodes for accessing deep brain regions in animal models with spatial and temporal resolution [1-3]. These minimally invasive probes can be simultaneously exploited in both optogenetics, for manipulation of neural activity, and for recording molecular and cellular activity in fiber photometry. It will be also shown how tapered fibers can be employed in-vivo in Raman and SERS spectroscopy experiments for tumoral tissue identification and label-free neurotechnology.

References

[1] B.Spagnolo et al., Tapered fibertrodes for optoelectrical neural interfacing in small brain volumes with reduced artefacts, Nature Materials, 21(7), 826–835, 2022.

[2] F.Pisano et al., (2019). Depth-resolved fiber photometry with a single tapered optical fiber implant, Nature Methods, 16(11), 2019.

[3] F.Pisanello et al., Dynamic illumination of spatially restricted or large brain volumes via a single tapered optical fiber,  Nature Neuroscience, 20(8), 1180–1188, 2017.